Yes, you can absolutely exclude external audio input from the system. Doing so shifts the instrument's classification from a Live Granular Processor to a Granular Synthesizer or Texture Generator.

Based on the provided sources, specifically the Daisy Field Block Diagram [Source 1113] and the DaisySP Reference [Source 555], here is how you can replace external input with internal sources while maintaining the "Texture Freeze" architecture.

### 1. Architectural Change: Replacing the Input Source

In the original design, the Circular Buffer records signals from the hardware audio input. To exclude external input, you simply reroute the signal flow to record from an internal Source Engine instead.

- Original Flow: Hardware In $\rightarrow$ Circular Buffer $\rightarrow$ Granular Engine

- New Flow: Internal Source $\rightarrow$ Circular Buffer $\rightarrow$ Granular Engine

Available Internal Sources (DaisySP):

- Oscillators: Use daisysp::Oscillator or daisysp::Fm2 to generate tonal drones that are written into the buffer [Source 1113].

- Noise/Aleatoric: Use daisysp::WhiteNoise or daisysp::Dust to create chaotic textures. This is ideal for "Clouds"-style synthesis where the granular engine processes noise into tonal washes [Source 555].

- Impulses: Use daisysp::KarpusStrong (Plucked String) algorithms as the source. The granular engine would then "freeze" the decay tail of a pluck, allowing you to stretch a percussive hit into an infinite pad [Source 738, 739].

### 2. Operational Changes (The "Freeze" Logic)

When using internal sources, the function of the "Freeze" (Write Disable) button changes slightly:

- With External Input: "Freeze" stops capturing the outside world.

- With Internal Input: "Freeze" acts as a "Resample Lock."

  - Unfrozen (Write Enabled): The buffer is constantly overwritten by the internal oscillator. The granular engine plays back the live synth voice (like a delay effect).

  - Frozen (Write Disabled): The buffer stops updating. The oscillator continues running in the background (inaudible), but the granular engine plays the captured segment of that oscillator. This allows you to capture a complex FM timbre and then scrub through it statically.

### 3. C++ Implementation Strategy

In your AudioCallback, you conditionally fill the buffer from a synth object instead of the in[i] array.

// Global Objects

daisysp::BlOsc oscillator; // Band-limited oscillator

daisysp::WhiteNoise noise;

void AudioCallback(float **in, float **out, size_t size) {

// ... controls processing ...

for (size_t i = 0; i < size; i++) {

// Generate Internal Source

float synth_signal = oscillator.Process() + (noise.Process() * 0.1f);

// Write to Freeze Buffer (Internal Signal instead of Input)

freezeBuf.Write(synth_signal);

// Granular Read (Same as before)

float gran_out = freezeBuf.Read(read_index);

out[i] = gran_out;

out[i] = gran_out;

}

}

### 4. Static Buffers (Pre-loaded Audio)

Alternatively, you can completely remove the Write() mechanism and treat the buffer as a Read-Only Wavetable.

- Implementation: Populate the buffer[] array during the Init() phase with a pre-calculated waveform or a sample loaded from the Daisy's SD card (using daisy::WavPlayer logic).

- Result: The instrument becomes a pure Wavetable Granulator. The "Freeze" button becomes redundant (the audio is always frozen), freeing up that button for other controls, such as triggering the sequencer or changing the "Window Shape" [Source 977, 981].

Here is the comprehensive Technical Design Report for the "Nebula-Resonator," refined to exclude external audio input in favor of internal synthesis sources, as requested.

# Technical Design Report: The "Nebula-Resonator"

A Standalone MIDI-Driven Granular Texture Synthesizer

Date: October 26, 2023 Platform: Daisy Pod / Daisy Field (libDaisy + DaisySP) Architecture: Internal Synthesis $\rightarrow$ Freeze Buffer $\rightarrow$ Granular Engine

## 1. Executive Summary

The "Nebula-Resonator" is a hybrid embedded instrument designed to generate evolving textural soundscapes. Unlike traditional granular processors that rely on live audio input, this system utilizes a self-contained synthesis engine (Virtual Analog and Noise) to feed a Circular Freeze Buffer.

The core innovation is the "Texture Freeze" mechanism: a decoupling of the buffer's Write Pointer from its Read Pointers. When "Frozen," the system captures a transient moment of the internal synth—such as a complex FM bell tone or a noise burst—and transforms it into a static wavetable. A synchronized Step Sequencer then modulates the granular scan position, animating the static texture rhythmically. This creates a "Rhythmic Drone" architecture suitable for IDM, ambient, and techno genres [Source 303, 546].

## 2. DSP Architecture

### 2.1 The Internal Source Engine

To replace external input, we implement a flexible sound generator block. This ensures the granular engine always has rich harmonic material to chew on.

- Oscillator: A band-limited oscillator (daisysp::BlOsc) providing Sawtooth and Square waves for harmonic content [Source 669, 1073].

- Texture Layer: A daisysp::WhiteNoise generator mixed in to add "grit" and spectral density, which results in "cloud-like" textures when granulated [Source 1064].

- Routing: Source = (Oscillator * Level) + (Noise * Level) $\rightarrow$ FreezeBuffer.

### 2.2 The "Freeze" Circular Buffer

The heart of the effect is a custom Circular Buffer implementation that supports conditional writing.

- Theory: Standard delay lines write continuously. The Freeze Buffer implements a boolean freeze_state. When true, the Write() function is bypassed, but the Read() function continues to access the memory array using modulo wrapping [Source 620, 622].

- Buffer Size: Allocated in SDRAM (on Daisy Field/Pod) to allow for 1–2 seconds of high-fidelity capture (e.g., 96,000 samples at 48kHz).

C++ Implementation Logic:

// Logic derived from Pirkle's Delay Line architectures [Source 621]

void ProcessFreezeBuffer(float input_sample) {

// 1. WRITE (Conditional)

if (!freeze_active) {

buffer[write_head] = input_sample;

write_head++;

if (write_head >= BUFFER_SIZE) write_head = 0;

}

// 2. READ (Always active)

// Granular read heads access 'buffer' independently of write_head state

}

### 2.3 The Multi-Tap Granular Engine

Instead of a single playback point, the system uses 4 Asynchronous Read Heads (Grains).

- Jitter: Each read head has a randomized offset (daisysp::Jitter) applied to its position. This prevents phase cancellation artifacts (comb filtering) and creates a diffuse stereo image [Source 1071, 302].

- Windowing: To prevent clicks when the read heads wrap around the buffer or jump positions, a Hanning window is applied to the grains [Source 231, 297].

## 3. Control Architecture: The Parameter Sequencer

### 3.1 Step Sequencer Logic

The internal sequencer does not play pitch; it plays Time. It modulates the Read Position of the granular engine.

- Structure: An 8-step array holding values 0.0 to 1.0.

- Operation: On every clock tick, the sequencer outputs a value that jumps the read heads to a specific region of the frozen buffer (e.g., Step 1 scans the "Attack" of the captured sound; Step 2 scans the "Decay").

- Smoothing: A daisysp::Line or Port filter smooths the transition between steps to prevent digital clicking during jumps [Source 1064, 1075].

### 3.2 MIDI Integration

The system synchronizes to external DAWs using MIDI Clock.

- Clock Handling: The system listens for MIDI Real-Time byte 0xF8 (Timing Clock).

- Division: It counts 6 ticks to advance a 16th note (24 PPQN standard) [Source 38, 1080].

- Gating: MIDI Note-On messages trigger the internal Source Engine envelope, effectively "injecting" a new sound into the buffer to be captured [Source 645, 648].

## 4. Interface & Mapping (Finite State Machine)

To manage complexity on the Daisy Pod/Field, we use the Paged Super-Controller design (FSM).

Global Controls:

- Button 1: FREEZE Toggle. (Locks the buffer content).

- Button 2: Tap Tempo / Sequencer Start.

| Page (LED Color) | Super Knob 1 Function | Super Knob 2 Function |
| --- | --- | --- |
| 1. SOURCE (Cyan) | **Timbre Morph:**0-50%: Osc Waveform (Saw->Sqr)50-100%: Adds Noise & Drive. | **Input Envelope:**Controls the Decay/Release of the internal source. Short clicks vs. Long drones. |
| 2. GRANULAR (Magenta) | **Texture/Jitter:**Increases grain density and randomizes playback position (Diffusion). | **Scan/Window:**Manual scrub through buffer (if Seq is off) OR Grain Size (Short/Glitchy -> Long/Pad). |
| 3. MOTION (Yellow) | **Seq Depth:**How strongly the sequencer modulates the scan position. | **Reverb/Space:**Global Reverb Mix and Stereo Widening. |

## 5. Implementation Roadmap

### Phase 1: The "Digital Twin" Test

Before flashing hardware, compile the logic as a VST/AU using the DaisySP wrappers.

- Test: Verify that the "Freeze" logic creates a clean loop without clicks.

- Test: Verify that the MIDI Clock parser correctly locks to a DAW tempo [Source 126, 128].

### Phase 2: Core C++ Class Structure

Create a Voice struct to encapsulate the logic:

struct NebulaVoice {

// DSP Objects

daisysp::BlOsc oscillator;

daisysp::WhiteNoise noise;

daisysp::AdEnv amp_env;

// Buffer

float* freeze_buffer;

bool is_frozen;

// Methods

void Init(float sample_rate, float* buf_ptr);

float Process(float knob_timbre, float knob_decay);

void Trigger(); // Fires the internal source

void ToggleFreeze();

};

### Phase 3: The Audio Callback

The main loop processes the internal source, writes to the buffer (if unfrozen), and then reads from the buffer using the multi-tap grains.

Key Challenges & Solutions:

- Click Removal: When "Freezing," the sudden stop of the write head can cause a click.

  - Solution: Implement a 10ms crossfade or "Wait for Zero Crossing" logic before engaging the freeze state [Source 263].

- Silence: If the buffer is frozen while empty, the output is silence.

  - Solution: Initialize the buffer with pre-computed noise or a sine wave during Setup() so the instrument makes sound immediately [Source 289].

## 6. References

- Circular Buffers & Freeze: Pirkle, Designing Audio Effect Plugins in C++ [Source 620, 621].

- Granular Synthesis: Miranda, Computer Sound Design [Source 214].

- MIDI Clock: Edstrom, Arduino for Musicians [Source 38].

- DaisySP Modules: BlOsc [Source 1073], Jitter [Source 1071].

Here is the Implementation Specification to accompany the Technical Design Report. This section provides the concrete C++ data structures and logic required to build the "Nebula-Resonator" on the Daisy platform, focusing on the custom "Freeze" buffer and the granular read engine.

# Implementation Specification: Nebula-Resonator

## 1. The Core DSP Classes

### 1.1 The FreezeBuffer Class

This class manages the memory. Unlike a standard DelayLine, it separates the Write logic from the Read logic to allow the "Freeze" state (stopping the write head while reading continues).

Key Logic:

- Write: Only increments the write index if frozen is false.

- Read: Always calculates the read index relative to the current write index to maintain continuity when freezing/unfreezing [Source 128, 622].

#include "daisysp.h"

// 2 seconds at 48kHz

#define BUFFER_SIZE 96000

class FreezeBuffer {

public:

float data[BUFFER_SIZE];

size_t write_pos;

bool is_frozen;

void Init() {

for(size_t i = 0; i < BUFFER_SIZE; i++) data[i] = 0.0f;

write_pos = 0;

is_frozen = false;

}

void SetFreeze(bool state) { is_frozen = state; }

// Conditional Write

void Write(float input) {

if (!is_frozen) {

data[write_pos] = input;

write_pos++;

if (write_pos >= BUFFER_SIZE) write_pos = 0;

}

}

// Fractional Read [Source 624]

float Read(float delay_in_samples) {

// 1. Calculate Read Position relative to Write Head

float read_idx_float = (float)write_pos - delay_in_samples;

// 2. Handle Wrapping (Circular Buffer Logic)

while(read_idx_float < 0.0f) read_idx_float += BUFFER_SIZE;

while(read_idx_float >= BUFFER_SIZE) read_idx_float -= BUFFER_SIZE;

// 3. Linear Interpolation

int32_t idx_int = (int32_t)read_idx_float;

float frac = read_idx_float - idx_int;

int32_t idx_next = (idx_int + 1) < BUFFER_SIZE ? idx_int + 1 : 0;

float val1 = data[idx_int];

float val2 = data[idx_next];

return val1 + frac * (val2 - val1);

}

};

### 1.2 The TextureVoice Class (Granular Engine)

This class generates the "Cloud." It utilizes 4 asynchronous read heads (taps) that jitter around a central scan position. This creates a thick, chorus-like texture from the static buffer.

Key Components:

- Jitter: Uses daisysp::Jitter to randomize the delay time of each head [Source 1067].

- Stereo Spread: Taps 1 & 3 go Left, 2 & 4 go Right.

using namespace daisysp;

class TextureVoice {

public:

Jitter jitter; // One randomizer for each tap

float tap_offsets; // Base spacing for the taps

void Init(float sample_rate) {

// Initialize Jitter modules for "Brownian" motion

for(int i=0; i<4; i++) {

jitter[i].Init(sample_rate);

jitter[i].SetCpsMin(0.5f);  // Slow drift

jitter[i].SetCpsMax(4.0f);  // Fast twitch

jitter[i].SetAmp(500.0f);   // Jitter range +/- 500 samples

}

// Spacing to prevent phasing

tap_offsets = 0.0f;

tap_offsets = 300.0f;

tap_offsets = 600.0f;

tap_offsets = 900.0f;

}

// Process: Returns Stereo Frame

void Process(FreezeBuffer& buf, float scan_pos_0to1, float texture_amt, float* outL, float* outR) {

float base_delay = scan_pos_0to1 * BUFFER_SIZE;

float accL = 0.0f;

float accR = 0.0f;

for(int i=0; i<4; i++) {

// Calculate randomized delay time

float jit_val = jitter[i].Process() * texture_amt;

float final_delay = base_delay + tap_offsets[i] + jit_val;

// Constrain delay to buffer limits

if(final_delay < 1.0f) final_delay = 1.0f;

if(final_delay > BUFFER_SIZE - 1) final_delay = BUFFER_SIZE - 1;

// Read from Freeze Buffer

float sig = buf.Read(final_delay);

// Pan: Evens to Left, Odds to Right

if(i % 2 == 0) accL += sig;

else accR += sig;

}

// Normalize output (divide by num taps/2)

*outL = accL * 0.5f;

*outR = accR * 0.5f;

}

};

## 2. Integration: The Audio Callback

This section demonstrates how to wire the Internal Source, the Sequencer, and the Texture Engine together within the libdaisy callback.

Control Mapping:

- Knob 1: Scan Position (Base).

- Sequencer: Modulates Scan Position.

- Knob 2: Texture Amount (Jitter intensity).

#include "daisy_pod.h"

#include "daisysp.h"

// Hardware and DSP Objects

daisy::DaisyPod hw;

FreezeBuffer freezeBuf;

TextureVoice cloud;

daisysp::BlOsc internalSource; // The internal synth [Source 1073]

daisysp::Metro clock;

daisysp::Port slew;            // For smoothing sequencer jumps [Source 1071]

// Sequencer Data

float seq_steps = { 0.1f, 0.2f, 0.5f, 0.1f, 0.8f, 0.9f, 0.4f, 0.2f };

int seq_idx = 0;

void AudioCallback(daisy::AudioHandle::InputBuffer in, daisy::AudioHandle::OutputBuffer out, size_t size) {

hw.ProcessAllControls();

// --- 1. READ CONTROLS ---

float knob1 = hw.knob1.Process(); // Scan Base

float knob2 = hw.knob2.Process(); // Texture Amount

// --- 2. HANDLE FREEZE BUTTON ---

// Button 1 Toggles Freeze

if(hw.button1.RisingEdge()) {

freezeBuf.SetFreeze(!freezeBuf.is_frozen);

}

// Set LED Red if Frozen, Green if Recording

hw.led1.Set(freezeBuf.is_frozen ? 1.0f : 0.0f, !freezeBuf.is_frozen ? 1.0f : 0.0f, 0.0f);

// --- 3. SEQUENCER LOGIC ---

if(clock.Process()) {

seq_idx = (seq_idx + 1) % 8;

}

// Smooth the sequencer step value to avoid clicks when jumping

float current_seq_val = slew.Process(seq_steps[seq_idx]);

// --- 4. AUDIO LOOP ---

for(size_t i = 0; i < size; i++) {

// A. Generate Internal Source (Sawtooth)

float synth_sig = internalSource.Process();

// B. Write to Buffer (if not frozen)

freezeBuf.Write(synth_sig);

// C. Calculate Scan Position

// Mix Knob 1 (Manual) + Sequencer (Auto)

float scan_pos = knob1 + (current_seq_val * 0.3f);

// Wrap 0-1

if(scan_pos > 1.0f) scan_pos -= 1.0f;

// D. Generate Texture

float wetL, wetR;

cloud.Process(freezeBuf, scan_pos, knob2, &wetL, &wetR);

// E. Output

out[i] = wetL;

out[i] = wetR;

}

}

int main(void) {

hw.Init();

hw.SetAudioBlockSize(4);

float sample_rate = hw.AudioSampleRate();

// Init DSP

freezeBuf.Init();

cloud.Init(sample_rate);

internalSource.Init(sample_rate);

internalSource.SetWaveform(daisysp::BlOsc::WAVE_SAW);

internalSource.SetFreq(110.0f); // Low drone A2

internalSource.SetAmp(0.5f);

clock.Init(4.0f, sample_rate); // 4 Hz Clock

slew.Init(sample_rate, 0.05f); // 50ms smoothing time

hw.StartAudio(AudioCallback);

while(1) {}

}

### Summary of Operation

- Internal Source: A sawtooth drone runs continuously.

- Recording: The FreezeBuffer captures this drone. When you press Button 1, recording stops, trapping the last 2 seconds of audio.

- Playback: The TextureVoice reads from the buffer.

- Motion: The Metro steps through the seq_steps array. This value passes through a Port (slew limiter) to smoothly glide the read heads to new positions in the buffer.

- Result: A rhythmic, shifting textural drone that can be manually scanned with Knob 1 or disintegrated with Knob 2.